Technical Report VGA

Name: Al-Moataz Billah Alaa El-Dien Mohammed Ramadan

# Introduction

Video Graphics Array (VGA) is an analog computer display standard introduced in 1987 by IBM, utilizing a 15-pin connector to transmit video signals. Although largely replaced by digital standards like HDMI and DisplayPort, it is still used for legacy hardware, supporting common resolutions up to 640 x 480 pixels.

# Discussion

Our project is designing the VGA controller with resolution 640 x 480 60Hz, so we have 640 pixel per line and 480 line per screen, and we want to display a video with 60 frame per second so the clock frequency will be

Fclk=800pixelline x 525lineframe x 60framesecond=25.2 MHz

∴T=40 ns

The CRT starts to shoot electrons to the screen line by line, it starts by the first line and then the second and so on, the screen has areas where the CRT doesn't shoot electrons to allow for CRT to retrace and start in the next line

This diagram shows the timing diagram of the signals and where the display happens and where the blanking timing occurs.

# Verilog code for design

module VGA_controller #(parameter h_active      = 640,

h_front_porch = 16,

h_sync_porch  = 96,

h_back_porch  = 48,

v_active      = 480,

v_front_porch = 10,

v_sync_porch  = 2,

v_back_porch  = 33,

BPP           = 8

)

(

input clk, rstn,

output reg [BPP-1:0] rgb,

output h_sync, v_sync, video_on

);

localparam h_sync_width = h_active + h_front_porch + h_back_porch + h_sync_porch;

localparam v_sync_width = v_active + v_front_porch + v_back_porch + v_sync_porch;

localparam ADDR_WIDTH   = $clog2(v_active * h_active);

localparam pixel_x_bits = $clog2(h_sync_width);

localparam pixel_y_bits = $clog2(v_sync_width);

wire [pixel_x_bits-1:0] pixel_x;

wire [pixel_y_bits-1:0] pixel_y;

wire tick_x, tick_y;

wire [ADDR_WIDTH - 1:0] addr;

modulus_counter #(.M(h_sync_width))

pixels_x (

.reset(rstn),

.clk(clk),

.enable(1'b1),

.count(pixel_x),

.done(tick_x)

);

modulus_counter #(.M(v_sync_width))

pixels_y (

.reset(rstn),

.clk(clk),

.enable(tick_x),

.count(pixel_y),

.done(tick_y)

);

// assuming that the photos that will be displayed will be exactly the same

// in wdith and height as the screen which is 640 x 480 and each pixel has

// a color and each color is represented with BPP bits so the word will have

// 8 bits for color, and each line will represent one pixel, and each pixel is

// represented by one line, so number lines equal to total number of pixels

// = 640 * 480 = 307200, this number will change if the pixels changes

reg [BPP-1:0] memory [0 : 2 ** ADDR_WIDTH - 1];

assign video_on = (pixel_x < h_active  && pixel_y < v_active)? 1'b1 : 1'b0;

assign h_sync = (pixel_x <(h_active + h_front_porch - 1) || pixel_x >=(h_active + h_front_porch + h_sync_porch - 1))? 1'b1 : 1'b0;

assign v_sync = (pixel_y <(v_active + v_front_porch - 1) || pixel_y >=(v_active + v_front_porch + v_sync_porch - 1))? 1'b1 : 1'b0;

// addr = number of columns * counter_of_rows + counter_of_columns

assign addr = v_active * pixel_y  + pixel_x;

always @(posedge clk)

begin

if(video_on)

rgb = memory[addr];

else

rgb = rgb;

end

endmodule

module modulus_counter #(parameter M = 800) (

input reset, clk, enable,

output [$clog2(M)-1:0] count,

output done

);

reg [$clog2(M)-1:0] q_reg, q_next;

always @(posedge clk, negedge reset)

begin

if(!reset)

q_reg <= 'b0;

else if(enable)

q_reg <= q_next;

else

q_reg <= q_reg;

end

assign done = (q_reg == M-1);

always @(*)

begin

if(done)

q_next = 'b0;

else

q_next = q_reg + 1;

end

assign count = q_reg;

endmodule

module VGA_tb;

// Parameters

localparam  h_active      = 640,

h_front_porch = 16,

h_sync_porch  = 96,

h_back_porch  = 48,

v_active      = 480,

v_front_porch = 10,

v_sync_porch  = 2,

v_back_porch  = 33,

BPP           = 8,

T             = 40;

// Ports

reg clk, reset;

wire [BPP - 1:0] rgb;

wire h_sync, v_sync, video_on;

VGA_controller # (

.h_active(h_active),

.h_front_porch(h_front_porch),

.h_sync_porch(h_sync_porch),

.h_back_porch(h_back_porch),

.v_active(v_active),

.v_front_porch(v_front_porch),

.v_sync_porch(v_sync_porch),

.v_back_porch(v_back_porch),

.BPP(BPP)

)

VGA_inst (

.clk(clk),

.rstn(reset),

.rgb(rgb),

.h_sync(h_sync),

.v_sync(v_sync),

.video_on(video_on)

);

always  begin

clk = 0;

#(T/2);

clk = 1;

#(T/2);

end

initial begin

reset = 0;

#2;

reset = 1;

$readmemb("data.mem", VGA_inst.memory);

#(10**9);

$stop;

end

endmodule

# Waveforms

This shows the video_on signal which is high when we are in the display area, to make sure, here is the calculation

display time=width of the high period of video on signalperiod of clock=25600 n40 n=640

Which is the same as the value of pixels per line as displayed in timing graph.

This photo shows that the h_sync signal is high for the intended period

width of hsync signal=2820040=705 s

width of the off period=380040=95 s

The signal lasts for the intended period as specified by the standards

These two images shows that v_sync signals goes high and low when pixel_y reach 491 and 489 respectively, which assures again that our design works as the standard specifies.

The video_on signal becomes low before h_sync becomes low by the amount of h_front_porch  h_front_porch= 64040=16